AUTONOMOUS CLOUD SOC Secure Wireless Networks Network Security Rubric 5 — WPA3 Configuration & WIPS Plan

| Document Title | Secure Wireless Networks — R5 |
| --- | --- |
| Student Name | Khushbu Jalan |
| Program | DAE Cybersecurity Program |
| Project | Autonomous Cloud SOC & Threat Intelligence System |
| Rubric Reference | Network Security Rubric 5 — Secure Wireless Networks |
| Document Version | 1.0 |
| Date | May 2026 |
| Status | COMPLETE — Written Policy & Configuration Document |

PROJECT CONTEXT: The Autonomous Cloud SOC core infrastructure runs entirely on virtual networks (UTM + Docker Desktop on MacBook). There is no wireless component in the SOC itself. However, the SOC is administered remotely over the administrator's WiFi network — making wireless security directly relevant to protecting the SOC management plane. This document covers the WPA3 configuration of the admin access WiFi network and a WIPS plan to prevent unauthorised access to the SOC management interface.

# Section 1 — Wireless Security Overview

## 1.1  Why Wireless Security Matters for This SOC

The Autonomous Cloud SOC is managed exclusively from a MacBook connected to a home/lab WiFi network. All administrative actions — accessing the Wazuh dashboard, configuring n8n workflows, reviewing Grafana alerts — are performed over this wireless connection. This means the WiFi network is the first line of defence for the SOC management plane. A compromised WiFi network gives an attacker the same network access as the SOC administrator.

Three specific threats make wireless security critical for this deployment:

- Eavesdropping: An attacker on the same WiFi network could capture unencrypted management traffic between the MacBook and the Docker-hosted SOC services.

- Rogue Access Point: An attacker could set up a fake WiFi network with the same SSID to intercept SOC admin credentials via an evil twin attack.

- Unauthorised Access: A neighbour or nearby attacker cracking weak WiFi credentials could gain access to the same network as the SOC and attempt lateral movement to the MacBook running the SOC stack.

## 1.2  Wireless Network — Configuration Summary

| Parameter | Configuration | Security Rationale |
| --- | --- | --- |
| Security Protocol | WPA3-Personal (SAE) | Replaces WPA2-PSK — resistant to offline dictionary attacks due to Simultaneous Authentication of Equals (SAE) handshake |
| Encryption | AES-256 (CCMP) | Military-grade encryption — replaces TKIP which is vulnerable to KRACK attack |
| WiFi Standard | 802.11ax (WiFi 6) | Latest standard — mandatory Management Frame Protection (MFP) prevents deauth spoofing |
| SSID Broadcast | Enabled — non-identifying name | SSID hidden provides false security; a non-identifying name is preferred |
| Band | 5 GHz primary | Shorter range reduces exposure to neighbouring attackers; less interference |
| Channel Width | 80 MHz | Balanced throughput and interference resistance |
| Router Admin Access | Local only — WiFi disabled | Router admin panel accessible only via Ethernet cable — not over WiFi |
| DNS | Encrypted DNS over HTTPS | Prevents DNS-based eavesdropping on SOC management queries |
| Guest Network | Isolated VLAN — no LAN access | Visitors cannot reach the MacBook SOC host from the guest network |

RUBRIC ITEM SATISFIED: WPA3 configuration for 1 network — COMPLETE

# Section 2 — WPA3 Configuration Details

## 2.1  WPA3-Personal — How It Works

WPA3-Personal uses the Simultaneous Authentication of Equals (SAE) protocol — a replacement for the WPA2 Pre-Shared Key (PSK) handshake. SAE provides two critical security improvements over WPA2:

| Feature | WPA2-PSK | WPA3-SAE |
| --- | --- | --- |
| Handshake type | 4-way handshake — capturable offline | SAE — forward secrecy, no offline crack possible |
| Dictionary attack risk | HIGH — capture handshake and crack offline | NONE — SAE prevents offline dictionary attacks |
| KRACK vulnerability | Vulnerable (CVE-2017-13077) | Not applicable — SAE replaces vulnerable handshake |
| Forward Secrecy | No — one key compromise = all sessions | Yes — unique session keys protect past sessions |
| Management Frame Protect | Optional | Mandatory — prevents deauth flood attacks |

## 2.2  Router Configuration Steps

The following steps were applied to configure WPA3 on the admin WiFi network used to access the SOC management plane:

Step 1 — Access Router Admin Panel

Connect via Ethernet cable (not WiFi) to prevent lockout. Open browser and navigate to the router's admin IP:

http://192.168.1.1

Log in with the router admin credentials. Default credentials were changed on first setup.

Step 2 — Navigate to Wireless Security Settings

Go to: Wireless Settings → Security → Authentication Method.

Step 3 — Set Security Protocol to WPA3

Change the Authentication Method dropdown from WPA2-Personal to:

WPA3-Personal  (or WPA2/WPA3 Transitional for older device compatibility)

WPA2/WPA3 Transitional mode allows older devices to connect with WPA2 while newer devices use WPA3 — providing maximum compatibility without sacrificing security for capable devices.

Step 4 — Set Encryption to AES

Encryption: AES (CCMP) — do NOT select TKIP

Step 5 — Set a Strong Passphrase

A strong WiFi passphrase was configured meeting the following requirements:

- Minimum 16 characters length

- Mix of uppercase, lowercase, numbers, and symbols

- Not a dictionary word or personal information

- Unique — not reused from any other account

Step 6 — Enable Management Frame Protection

Management Frame Protection (MFP / PMF): Required

Setting MFP to Required (not Optional) ensures all management frames are cryptographically protected — preventing deauthentication flood attacks that force devices to reconnect and expose handshakes.

Step 7 — Configure Guest Network Isolation

A separate guest SSID was configured on an isolated VLAN. Guest network settings:

Guest SSID:   SOC-GUEST

Security:     WPA3-Personal

VLAN:         Isolated — AP isolation enabled

LAN Access:   Disabled — guests cannot reach 192.168.1.0/24

Step 8 — Disable WPS

WiFi Protected Setup (WPS) was disabled. WPS PIN attacks (Pixie Dust) can crack WPS in minutes regardless of WPA3 password strength.

WPS: Disabled

RUBRIC ITEM SATISFIED: WPA2/WPA3 configuration for 1 network — COMPLETE

# Section 3 — Wireless Intrusion Prevention System (WIPS) Plan

## 3.1  What is WIPS

A Wireless Intrusion Prevention System (WIPS) monitors the wireless spectrum for unauthorised access points, rogue clients, and attack signatures. It detects threats that operate at the RF layer — below the network layer — which traditional firewalls and SIEM tools cannot see.

## 3.2  Threats WIPS Detects and Prevents

| Threat | Description | WIPS Response |
| --- | --- | --- |
| Rogue Access Point | Unauthorised AP connected to the LAN mimicking legitimate SSID | Detect via MAC/SSID mismatch — alert SOC operator — block on switch port |
| Evil Twin Attack | Attacker creates fake AP with same SSID to capture credentials | Detect via signal strength anomaly and BSSID mismatch — deauth clients from rogue AP |
| Deauth Flood Attack | Attacker sends spoofed deauthentication frames to disconnect clients | MFP (WPA3) blocks spoofed frames — WIPS detects flood pattern and alerts |
| Unauthorised Client | Unknown device connects to the network without authorisation | MAC-based detection — alert generated — quarantine or block applied |
| WPS Brute-Force | Attacker uses Pixie Dust or PIN brute-force against WPS | WPS disabled — WIPS detects WPS probe floods and blocks source |
| Packet Injection Attack | Attacker injects crafted frames to disrupt or intercept traffic | WIPS detects malformed frame patterns — alert generated |

## 3.3  WIPS Implementation — Kismet

For this SOC project, Kismet is selected as the WIPS solution. Kismet is an open-source wireless network detector, sniffer, and intrusion detection system that runs on Linux — making it compatible with the Kali Linux VM already deployed in this project.

| Parameter | Configuration |
| --- | --- |
| Tool | Kismet — open source WIPS/WIDS |
| Platform | Kali Linux VM (UTM on MacBook) |
| Monitor Interface | wlan0mon — wireless adapter in monitor mode |
| Detection Scope | 2.4 GHz and 5 GHz bands |
| Alert Types | Rogue AP, Evil Twin, Deauth Flood, WPS Probe, Packet Injection |
| Log Format | JSON logs compatible with Wazuh log ingestion |
| Integration | Kismet JSON alerts → Wazuh → n8n → SOC operator notification |
| Dashboard | Kismet web UI on port 2501 — localhost only |

## 3.4  Kismet Installation & Configuration

Step 1 — Install Kismet on Kali

sudo apt update && sudo apt install kismet -y

Step 2 — Enable monitor mode on wireless adapter

sudo ip link set wlan0 down

sudo iw dev wlan0 set type monitor

sudo ip link set wlan0 up

# Verify monitor mode

iwconfig wlan0

Step 3 — Start Kismet

sudo kismet -c wlan0

Kismet starts capturing and analysing all wireless frames in range. Access the web dashboard at:

http://localhost:2501

Step 4 — Configure Kismet alerts for SOC integration

Edit the Kismet alert configuration to forward alerts to Wazuh:

# /etc/kismet/kismet_alerts.conf

alert=APSPOOF,5/min,10/min    # Evil twin detection

alert=DEAUTHFLOOD,5/min,10/min # Deauth flood detection

alert=WPSBRUTE,3/min,5/min    # WPS brute force detection

alert=ROGUE,1/min,5/min       # Rogue AP detection

Step 5 — Forward Kismet logs to Wazuh

Configure Wazuh agent on Kali to read Kismet JSON logs:

# /var/ossec/etc/ossec.conf — add localfile block

<localfile>

<log_format>json</log_format>

<location>/var/log/kismet/kismet.json</location>

</localfile>

RUBRIC ITEM SATISFIED: WIPS implementation plan to prevent unauthorised access — COMPLETE

# Section 4 — Wireless Security Policy

## 4.1  SOC Wireless Access Policy

The following policy governs all wireless access to the Autonomous Cloud SOC management network:

| Policy Rule | Requirement | Enforcement |
| --- | --- | --- |
| WPA3 mandatory | All SOC admin WiFi connections must use WPA3-Personal or WPA3-Enterprise | Router configured — WPA2-only devices blocked |
| Passphrase rotation | WiFi passphrase must be rotated every 90 days | Calendar reminder — immediate rotation after any suspected compromise |
| Approved devices only | Only registered MAC addresses permitted on SOC admin SSID | Router MAC filter — unknown MACs blocked |
| Guest isolation | Visitors must use isolated guest SSID — no LAN access | VLAN configured — AP isolation enabled |
| VPN on public WiFi | SOC must never be managed from public WiFi without VPN | Policy control — Tailscale VPN required for remote access |
| Router admin via cable | Router admin panel only accessible via Ethernet — WiFi admin disabled | Router setting — WiFi admin access disabled |
| WPS disabled | WPS must remain disabled at all times | Router setting — WPS off |
| WIPS monitoring | Kismet WIPS must run during all active SOC management sessions | Runbook requirement — WIPS started before admin session |

## 4.2  Incident Response for Wireless Threats

If Kismet WIPS detects any of the following alerts, the SOC operator must follow the response steps:

| WIPS Alert | Immediate Action | Follow-Up |
| --- | --- | --- |
| Rogue AP detected | Disconnect all clients — identify physical AP location | Remove rogue AP — investigate who connected it — update policy |
| Evil Twin detected | Warn all users — do not connect to unknown SSIDs | Change WiFi passphrase — check for credential compromise |
| Deauth flood | Check MFP enabled — identify attacker MAC | Block attacker MAC — review logs for data exfiltration |
| Unknown client | Block MAC immediately — check for data access | Review DHCP logs — change passphrase if breach confirmed |

# Section 5 — Rubric Compliance & Document Control

## 5.1  Rubric 5 Compliance Checklist

| Rubric Requirement | Where Addressed | Status |
| --- | --- | --- |
| WPA3 configuration for 1 network | Section 2 — full 8-step config | COMPLETE |
| WPA2/WPA3 transitional mode explained | Section 2.2 Step 3 | COMPLETE |
| WIPS to prevent unauthorised access | Section 3 — Kismet WIPS plan | COMPLETE |
| WIPS threat detection coverage | Section 3.2 — 6 threat types | COMPLETE |
| WIPS implementation steps | Section 3.4 — install + config steps | COMPLETE |
| Wireless security policy | Section 4.1 — 8 policy rules | COMPLETE |
| Wireless IR procedure | Section 4.2 — 4 threat responses | COMPLETE |

## 5.2  Document Control

| Author | Khushbu Jalan |
| --- | --- |
| Program | DAE Cybersecurity Program |
| Version | 1.0 |
| Semester | Semester 1 — Written Policy & Configuration Document |
| Next Review | Start of Semester 2 — prior to live deployment |
| GitHub Path | docs/R5_Secure_Wireless_Networks.docx |
| Git Commit Tag | git commit -m 'R5 COMPLETE — Secure Wireless Networks' |